Design and Verification of an Application Specific Integrated Circuit (ASIC)

Senior Project II

Kevin Cao, Whitley Forman, Dhruvit Naik,

Zachary Nelson* and Julie Swift

*Team Leader

Dr. Orlando Hernandez and Dr. Larry A. Pearlstein

May 2016

Fulfillment Page

Submitted to the Faculty of the Electrical and Computer Engineering Department

of The College of New Jersey

Written and Researched by:

Kevin Cao

Whitley Forman

Dhruvit Naik

Zachary Nelson

Julie Swift

In partial fulfillment of the Bachelor of Science degree in Electrical and Computer Engineering

Acknowledgements

Zach

Abstract

The goal of this project is to gain experience in VLSI design by designing a chip that will process digital streaming audio data. More specifically, we will implement a 512-tap digital finite impulse response (FIR) filter, which will be applied to an input stream in order to create an output stream. We used the I2C protocol to allow a host to control the chip and the serial I2S protocol for transferring digital audio streams in and out.

Our hardware design was represented using Verilog register-transfer level (RTL) code. Development has been done using Xilinx ISE Design Suite 14.7. Test-benches were also designed and implemented using Verilog. The end goal of the project is to implement the design on a field-programmable gate array (FPGA). We will bring up our FPGA design with a realistic environment including an audio source, audio sink, and a microcontroller for reading and writing registers. We also plan to send a simple CMOS integrated circuit design for fabrication by MOSIS that will help us gain experience in physical chip design and prepare future groups to fabricate our full design.

Keywords: Application-specific integrated circuit (ASIC), Very large scale integration (VLSI) I2S, I2C, Digital filtering

Table of Contents

List of Tables	4

List of Illustrations	5

Nomenclature	6

Introduction	7

Specifications	10

Chapter 1: Background	11

Chapter 2: System Design	11

Chapter 3: I2S Interface	13

Chapter 4: Digital Filtering	26

Chapter 5: Register Block	33

Chapter 6: I2C Interface	41

Chapter 7: Simulation/Verification	11

Chapter 8: FPGA Implementation	11

Chapter 9: EDA Tools and Physical Design	11

Chapter 10: Conclusion	57

References	58

Appendix A: Project Overview	59

Biography	59

Engineering Standards and Realistic Constraints Form	59

Engineering Standards, Speciciations, and Codes	59

Modern Engineering Tools	59

Appendix B: Management	61

Schedule	59

Meeting Minutes	59

List of Contacts	59

Material List	59

Financial Budget	59

Appendix C: Source Code	81

Appendix D: Test Benches	125

Appendix E: Industry Specifications	125

List of Tables

Table 1.1: Comparison of MOSIS Academic Account Types	12

Table 2.1: Interface Signals for I2S Input	16

List of Illustrations

Figure I.1: Process of Designing a System on a Chip	7

Figure 1.1: Multi-Project Wafer	11

Nomenclature

ASIC: Application Specific Integrated Chip

BIST: Built in Self-Test

CIF: Caltech Intermediate Form

DIP: Dual in-line package

EDA: Electronic Design Automation

FPGA: Field Programmable Gate Architecture

FSM: Finite-State Machine

GDSII: Graphic Data System II

I2C: Inter-Integrated Circuit

I2S: Integrated Interchip Sound

IC: Integrated Circuit

LSB: Least Significant Bit

MEP: MOSIS Educational Program

MPW: Multi-Project Wafer

MSB: Most Significant Bit

OCP: Open Cavity Plastic

PCB: Printed Circuit Board

RO: Read Only

RTL: Register-Transfer Level

RTR: Ready to Receive

RTS: Ready to Send

RW: Read/Write

WO: Write Only

XFC: Transfer Complete

VLSI: Very Large Scale Integration

Introduction

Zach

Specifications

General:

I2S:

Filter:

I2C:

Register:

Chapter 1: Background – Z. Nelson

Chapter 2: System Design – Z. Nelson

Chapter 3: I2S Interface – K. Cao

Chapter 4: Digital Filtering – D. Naik

Chapter 5: Register Block – J. Swift

Chapter 6:  I2C Slave Interface – W. Forman

Chapter 7: Simulation/Verficiation – K. Cao and W. Forman

Chapter 8: FPGA Implementation - Zachary Nelson

Chapter 9: EDA Tools and Physical Design – D. Naik

and J. Swift

Chapter 10: Conclusion

References

[1]   Mosis.com, ‘About Us’, 2015. [Online]. Available: https://www.mosis.com/what-is-mosis. [Accessed: 11 November 2015].

[2]  nxp.com,  ‘I2C-bus specification and user manual - UM10204’, 2015. [Online]. Available: http://www.nxp.com/documents/user_manual/UM10204.pdf. [Accessed: 10 November 2015]

Appendix A: Project Overview

Biography:

- Kevin Cao

  - Kevin is from Morris Plains, NJ and is a Computer Engineering major who is planning to enter the workforce after graduating at TCNJ. Kevin has interned as a software engineer at LGS Innovations, located in Florham Park, NJ.

- Whitley Forman

  - Whitley is from Ocean Grove, NJ and is an Electrical Engineering major who is continuing his education in the electrical field and will be obtaining his Master Electrician’s license after graduation. He is planning on using his new knowledge and experience with his current business to expand into new ventures.

- Dhruvit Naik

  - A resident of Mount Laurel, NJ, Dhruvit is a senior Computer Engineering major at TCNJ. He plans on entering the workforce after graduation and continuing his education in the coming years. He is the Vice-President of a startup, ThinkSOAS, INC.

- Zachary Nelson

  - From Cream Ridge, NJ, Zachary is a Computer Engineering major who is planning on attending graduate school after graduation. He has experience as a software engineering intern at Teletronics Technology Corporation and an undergraduate student researcher at TCNJ as part of the MUSE program.

- Julie Swift

From Robbinsville, NJ Juliann is a Computer Engineering major who is planning on entering the workforce after college. She has experience as a AutoCAD Designer interning at Linearization Technology and a Software Development Life Cycle Analyst interning at Educational Testing Services. She participated in the undergraduate student research program, MUSE, at TCNJ.

Engineering Standards and Realstic Contraints Form:

Engineering Standards, Specfications, and Codes:

Modern Engineering Tools:

Appendix B: Management

Schedule:

Meeting Minutes:

List of Contacts:

Material List:

Financial Budget:

| Item | Cost | Comment |
| --- | --- | --- |
| ISE Design Suite 14.7 | $0.00 | Free software |
| CORE 9 University | $0.00 | Vitech’s CORE in the Classroom program |
| Dropbox | $0.00 | Free up to 2GB of storage |
| Git/GitHub Desktop | $0.00 | Free software |
| Microsoft Project 2013 | $0.00 | Free from the Computer Science Department’s DreamSpark Program |
| Mentor Graphics | $0.00 | TCNJ Subscription |
| Linux RedHat Operating System | $0.00 | TCNJ Subscription |
| Nexys 4 Artix-7 FPGA Board | $192.41 | Purchased from Digilent |
| UDA 1380 Board | $20.00 | Needs to be purchased |
| Crystal Oscillators | $20.00 | Needs to be purchased |
| CY8CKIT-050 PSoC 5LP Development Kit | $0.00 | School already owns |
| Total Budget | $500.00 |  |
| Total Costs | $232.41 |  |
| End Balance | $267.59 | Well under-budget |

Appendix C: Source Code

Appendix D: Test Benches

Appendix E: Industry Specficiations